Density Measurement of Rock & Soil Samples

Laboratory Experiment — Field Sample Analysis

# 1. Objective

To determine the bulk density (g/cm³) of various geological materials (rock/soil samples) collected from the mine site using a cup displacement method.

# 2. Apparatus & Materials

•  Measuring cup (Vcup = 190 cm³)

•  Weighing balance (mass in grams)

•  Water (density = 1 g/cm³, i.e., 1 gm = 1 cm³)

•  Rock/soil samples: Topsoil, Powdered Pebble, Pebble, White Sandstone, Red Sandstone, Carb Shale, Coal, Clay

# 3. Cup Volume Determination

The volume of the cup was determined by filling it with water to the brim and measuring the mass of water displaced:

| Measurement | Value |
| --- | --- |
| Mass of empty cup (W₁) | 130 g |
| Mass of cup + water to brim (W₂) | 327 g |
| Mass of water = W₂ − W₁ | 190 gm |
| Cup volume (Vcup) | 190 cm³ |

Note: Since water density = 1 g/cm³, mass of water (g) = volume of water (cm³) = cup volume.

# 4. Theory & Key Formulae

## 4.1 Definitions

FF (Fill Factor) = ratio of solid volume to cup volume:

FF = Vsolid / Vcup

**For this experiment, FF = 1 (the cup is assumed to be fully packed with solid material).

## 4.2 Solid Volume

Vsolid = Vcup × FF

With FF = 1:   Vsolid = 190 cm³

## 4.3 Density Formula

ρ = Msolid / Vsolid = Msolid / (FF × Vcup)

For FF = 1 and Vcup = 190 cm³:

ρ (g/cm³) = Msolid (g) / 190 cm³

## 4.4 Void Volume

Vvoid = Vcup − Vsolid

## 4.5 Determining Cup Volume from Water

Vcup = W₂ − W₁

where W₁ = weight of empty cup, W₂ = weight of cup filled with water to brim (1 g water = 1 cm³)

# 5. Raw Mass Measurements

Each material was filled into the cup and weighed. Solid mass was obtained by subtracting the cup mass (130 g):

| Material | M(solid+cup) [g] | Cup Mass [g] | M(solid) [g] |
| --- | --- | --- | --- |
| Topsoil | 512 g | 130 g | 382 g |
| Powdered Pebble | 516 g | 130 g | 386 g |
| Pebble | 625 g | 130 g | 495 g |
| White Sandstone | 476 g | 130 g | 346 g |
| Red Sandstone | 483 g | 130 g | 353 g |
| Carb Shale | 511 g | 130 g | 381 g |
| Coal | 340 g | 130 g | 210 g |
| Clay | 413 g | 130 g | 283 g |

# 6. Results — Density Summary

Calculated density for each material using ρ = m / 190 cm³ (FF = 1):

| Material | Solid Mass (g) | ρ Measured (g/cm³) | ρ Calculated (g/cm³) |
| --- | --- | --- | --- |
| Topsoil | 382 g | 2.01 | 2.01 g/cm³ |
| Powdered Pebble | 386 g | 2.03 | 2.03 g/cm³ |
| Pebble | 495 g | 2.60 | 2.61 g/cm³ |
| White Sandstone | 346 g | 1.82 | 1.82 g/cm³ |
| Red Sandstone | 353 g | 1.85 | 1.86 g/cm³ |
| Carb Shale | 381 g | 2.00 | 2.01 g/cm³ |
| Coal | 210 g | 1.10 | 1.11 g/cm³ |
| Clay | 283 g | 1.29 | 1.49 g/cm³ |

Note: ρ Measured values are from experimental observations. ρ Calculated = m / 190.

# 7. Individual Sample Calculations

## Topsoil

M(solid + cup) = 512 g

M(solid) = 512 − 130 = 382 g

ρ = 382 / 190 = 2.01 g/cm³

## Powdered Pebble

M(solid + cup) = 516 g

M(solid) = 516 − 130 = 386 g

ρ = 386 / 190 = 2.03 g/cm³

## Pebble

M(solid + cup) = 625 g

M(solid) = 625 − 130 = 495 g

ρ = 495 / 190 = 2.60 g/cm³

## White Sandstone

M(solid + cup) = 476 g

M(solid) = 476 − 130 = 346 g

ρ = 346 / 190 = 1.82 g/cm³

## Red Sandstone

M(solid + cup) = 483 g

M(solid) = 483 − 130 = 353 g

ρ = 353 / 190 = 1.85 g/cm³

## Carb Shale

M(solid + cup) = 511 g

M(solid) = 511 − 130 = 381 g

ρ = 381 / 190 = 2.00 g/cm³

## Coal

M(solid + cup) = 340 g

M(solid) = 340 − 130 = 210 g

Vₛₒₗᵢᵈ = Vₜᵤₚ × FF = 190 cm³

ρ = 210 / 190 = 1.10 g/cm³

## Clay

M(solid + cup) = 413 g

M(solid) = 413 − 130 = 283 g

ρ = 283 / 190 ≈ 1.49 g/cm³  [Observed: 1.29 g/cm³]